## “In-Close” Billet License Plate

## Holder - Dyna

## Instructions

Part # CV-4605 Series

Fits 06 & 07 Dyna

Parts included:

1 – License plate frame

1 – Backing plate

1 – Mounting axle block

3 – ¼-20 x ½” counter sunk black screws

4 – 10/24 x ⅜” button head screws

Installation Time:	Approximately 1 hour or less.

- Use Locktite on all fasteners.

- Remove axle nut and washer.

- Install new mounting bracket. Reinstall axle nut, torque to factory specs and install safety clip.

- Check axle adjustment.

- Install license plate back to mount using ¼-20 x ½” screws.

- Install license plate frame using 10/24 x ⅜” button head screws.

- Before turning the rear wheel, check the clearance between the “In Close” holder and the pulley.

- Rotate the wheel before letting the bike down to double check clearances.

- And again, use Locktite on all fasteners.

Our Warranty Policy

Cycle Visions’ obligation under “Our Warranty Policy” is limited to one (1) year from date of purchase to repair or replacement of the defective item (s).  Chrome, polished, powder-coated, anodized finishes, or electrical parts are limited to thirty (30) day from date of purchase.

This warranty will be void on all products that show evidence of misapplication, improper installation, abuse, lack of proper maintenance, negligence, or alteration from their original design.

The consumer will be responsible for removing from the vehicle any defective item (s), transportation costs prepaid and for reinstallation upon return by Cycle Visions.  The loss of use of the product, loss of time, inconvenience, commercial loss or consequential damages is not covered.  Cycle Visions reserves the right to change the design of any product without assuming any obligation to modify any product previously manufactured.  Warranty returns of Cycle Visions products are few because of our high quality products.

Carefully inspect all parts…. If you find that your parts arrived in damaged conditional all original packaging material (box and other materials) must be saved to make a proper damage claim within seven (7) days.  You must be able to present the damaged boxes and materials to the freight carrier to verify your claim.  The user shall assume all legal, personal, injury risk and liability and all other obligations, duties and risks associated therewith.

# Cycle Visions

San Diego, California (619) 295.7800

www.cyclevisions.com

See our other products on the web!

Rev. 12/05							                                                       Made in the USA