SAFETY DATA SHEET

Section 1. Chemical Product and Company Identification

Product Name:     	Floc-Off™ Patent Pending

Product Use: 	Polymer Absorption/Deactivation

Supplier’s Name:	Innovative Next Generation Products, Inc.

Emergency Telephone Number: 	262-789-7730

Address:				P.O. Box 510408

New Berlin, WI  53151

Website:				www.flocoff.com

Section 2. Hazard(s) Identification

Hazard Statement:	Harmful if inhaled through prolonged or repeated exposure.

Precautionary Statement(s):	Do not breath dust/vapors. In case of inadequate ventilation, wear Personal Protective Equipment (PPE).	Wash hands after handling. Store away from incompatible materials. Dispose of waste and residues in accordance with local authority requirements.

Hazards Not Otherwise	None known.

Classified:

Section 3. Composition/Hazardous

Comments:	If chemical identity and/or exact percentage of composition have been withheld, this information is considered to be a trade secret.

| Chemical Name | CAS Number | % |
| --- | --- | --- |
| Sodium Chloride | 7647-14-5 | 29.2-29.70 |
| Calcium Phosphate Tribasic | 1306-06-5 | .194-1.11 |
| Perlite - Expanded | 93763-70-3 | 13.5-15.1 |
| This product may contain greater than 0.1% Quartz | 14808-60-7 |  0.1 |
| Potato Granules |  |  |

Section 4. First Aid Measures

Inhalation:	Remove the affected person to fresh air. Get medical attention if symptoms develop or persist.

Eyes:	Flush with water.  Get medical attention if irritation develops or persists.

Skin:	Wash and rinse skin thoroughly. Get medical attention if irritation develops or persists.

Ingestion:	Drink plenty of water. Get medical attention if symptoms develop or persist.

Section 5. Fire Fighting Measures

Flammability of the Product:	Not flammable.

Suitable Extinguishing Media:	Use extinguishing media suitable to surrounding fire.

Specific Hazards Arising from	During fire, gases hazardous to health may be formed.

the Product:

Protective Equipment:	If product is involved in a fire, wear full protective clothing including a positive-pressure, NIOSH approved, self-contained breathing apparatus.

Section 6. Accidental Release Measures

Personal Precautions:	Use appropriate Personal Protective Equipment (PPE).

Methods for Cleaning Up:	Contain spills to vacuum for proper disposal. Use a vacuum cleaner equipped with a HEPA filter.

Section 7. Handling and Storage

Handling:	Provide appropriate exhaust ventilation at places where dust is formed. Do not get in eyes. Wash thoroughly after handling. Do not ingest. Avoid breathing vapors, mist or dust.

Storage:	Avoid humid or wet conditions as product will cake and become hard. Keep container securely closed when not in use. Store away from incompatible materials (see Section 10). Keep away from sources of ignition. Becomes hygroscopic at 65-75% relative humidity.

Section 8. Exposure Controls/Personal Protection

Occupational Exposure Limits:	None.

Biological Limit Values:	None.

Appropriate Engineering Controls:	Ventilation should be sufficient to effectively remove and prevent buildup of any dusts or fumes that may be generated during handling.

Personal Protection:

Eyes:	Unvented, tight fitting goggles should be worn in dusty areas.  Maintain eyewash fountain in work area.

Skin:	Wear appropriate chemical resistant gloves. Wear suitable protective clothing.

Respiratory:	Use a NIOSH approved respirator is there is a risk of exposure to dust/fume at levels exceeding exposure limits. If engineering controls do not maintain airborne concentrations below recommended exposure limits or to an acceptable level, an approved respirator must be worn.

Section 9. Physical and Chemical Properties

Physical State and Appearance:	Powder, White/Yellow

Odor:	Mild

Odor Threshold:	N/A

pH:	6.5-7.5

Melting/Freezing Point:	1473.8  F (801 C)

Initial Boiling Point/Boiling Range:	2669  F (1465  C) (760 mmHg)

Flash Point:	N/A

Evaporation Rate:	N/A

Flammability (solid, gas):	N/A

Flammability Limits:	N/A

Vapor Pressure:	2.4 mm Hg (1376.6 F (747 C))

Vapor Density:	N/A

Relative Density:	2.16 (H2O = 1)

Water Solubility(ies):	26.4%

Partition coefficient: n-octanol/water:	N/A

Auto-ignition Temperature:	N/A

Decomposition Temperature:	N/D

Viscosity:	N/A

Section 10. Stability and Reactivity

Chemical Stability:			Stable at normal conditions.

Possibility of Hazardous Reactions:	None known.

Incompatibility with Various		Strong Acids. May become corrosive to metals when wet.

Substances:

Hazardous Decomposition 		May evolve chlorine gas when in contact with strong acids.

Reactions:

Section 11. Toxicological Information

Inhalation:	Irritation of the respiratory tract.

Eyes:	Irritation with burning and tearing.

Skin:	May cause irritation if contact made to damaged skin.

Ingestion:	Less than a few grams would not be harmful. For larger quantities drink plenty of water.

Acute Toxicity:

| Chemical Name | Exposure | Type of Effect | Concentration | Species |
| --- | --- | --- | --- | --- |
| Sodium Chloride | Oral | LD50 | 3000 mg/kg | Rat |
| Potato Granules | Oral | LD50 | 2000 mg/kg |  |
| Potato Granules | Dermal | LD50 | 2000 mg/kg |  |
| Potato Granules | Inhalative | LD50 | Dust: 5 mg/ ¼ h |  |

Carcinogenicity Category:

This product contains some crystalline silica (CS), which is considered a hazard by inhalation.  IARC has classified CS as carcinogenic for humans (Group 1).  CS is listed by NTP as a known human carcinogen. CS is also a known cause of silicosis, a non-cancerous lung disease.

Section 12. Ecological Information

Exotoxicity:

LC50 (fish, 96 h): 100mg/l

EC50 (daphnia, 48 h): 100 mg/l

IC50 (algae, 72 h): 100 mg/l

This product is not classified as environmentally hazardous.

Section 13. Disposal Considerations

There may be additional local or provincial requirements relating to the disposal of waste. Consult provincial or local regulations regarding the proper disposal of this material.

Section 14. Transport Information

DOT					Not classified as dangerous

IATA					Not classified as dangerous

IMDG					Not classified as dangerous

Section 15. Regulatory Information

US Federal Regulations		This product is considered hazardous as defined by the OSHA Hazard Communication Standard, 29 CFR 1910.1200. See Section 3.

TSCA	This material is listed in the TSCA inventory, and is not otherwise regulated by TSCA.

RCRA	This material is not defined as hazardous waste per 40 CFR 261.

CERCLA	Material is not reportable under CERCLA, local requirements may vary.

SARA	311/ 312 Hazard Categories - Immediate and Delayed – None

313 Reportable Ingredients – None

State Regulations:

California Proposition 65	This product contains chemicals known to the state of California to cause cancer.

International:

Canada	Listed on the Domestic Substances List (DSL)

Europe 	Listed on European Inventory of Existing Commercial Chemical Substances (EINECS)

Section 16. Other Information

HMIS Rating:	Health: 			1

Flammability:		0

Physical Hazard:	0

PPE:			X

Notes:	The PPE rating depends on circumstances of use.  See Section 8 for recommended PPE.

The Hazardous Material Information System (HMIS) is a voluntary, subjective alphanumeric symbolic system for recommending hazard risk and personal protection equipment information.  It is a subjective rating system based on the evaluator’s understanding of the chemical associated risks.  The end-user must determine if the code is appropriate for their use.

Disclaimer

Information given herein is offered in good faith as accurate, but without guarantee. Conditions of use and suitability of the product for particular uses are beyond our control: all risks of use of the product are therefore assumed by the user and we expressly disclaim all warranties of every kind and nature, including warranties of merchantability and fitness for a particular purpose in respect to the use or suitability of the product. Nothing is intended as a recommendation for uses, which infringe valid patents, or as extending license under valid patents. Appropriate warnings and safe handling procedures should be provided to handlers and users.