| 1.3 | Details of the supplier of the safety data sheet |  |  |
| --- | --- | --- | --- |
| Bomar Ltd, Sea Road, Kilcoole, Co. Wicklow, Ireland |  |  |  |
| 1.4 Emergency Telephone number |  |  | +353 (0)1 2875110 |

## 01. IDENTIFICATION OF THE SUBSTANCE/PREPARATION & THE COMPANY/UNDERTAKING

| 1.1 | Product Identifier |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Product Name |  |  | Juniper Oil |  |  |  |
| Biological Definition |  |  | Juniperus Communis Fruit Oil is the volatile oil obtained from the berries of the Juniper, Juniperus communis L., Cupressaceae. |  |  |  |
| INCI Name |  |  | Juniperus Communis Fruit Oil |  |  |  |
| Synonyms & Trade Names |  |  | - |  |  |  |
| CAS-No |  | 8002-68-4 / 73049-62-4 / 84603-69-0 | EC No. | - / - / 283-268-3 | EINECS No. | - / - / 283-268-3 |
| 1.2 | Relative identified uses of the substance or mixture and uses advised against |  |  |  |  |  |
|  | Suitable for cosmetic, flavour, fragrance and professional applications only. |  |  |  |  |  |

## 02. HAZARDS IDENTIFICATION

| 2.1 | Classification of the substance or mixture |  |  |
| --- | --- | --- | --- |
|  | The full text for all Hazard Statements displayed in Section 16. |  |  |
| Classification (EC 1272/2008) |  |  |  |
|  | H226 Flammable liquid and vapour H302 Acute toxicity oral, Category 4 H312 Acute toxicity dermal, Category 4 H332 Acute toxicity inhalation, Category 4 H315 Skin irritation, Category 2 H319 Eye irritation, Category 2A H317 Skin sensitisation, Category 1 H304 Aspiration hazard, Category 1 H400 Environmental hazard, Acute, Category 1 H410 Environmental hazard, Chronic, Category 1 |  |  |
| 2.2 |  | Label Elements |  |
| Label in accordance with (EC) No 1272/2008 |  |  |  |
|  | GHS08 GHS07 GHS09 GHS02 |  |  |
| Signal Word |  |  | Danger |
| Contains |  |  | ALPHA PINENE, BETA PINENE, LIMONENE |
| Hazard Statements |  |  |  |
|  | H226 Flammable liquid and vapour. H226 Flammable liquid and vapour H302 Harmful if swallowed H312 Harmful in contact with skin H332 Harmful if inhaled H315 Causes skin irritation H319 Causes serious eye irritation H317 May cause an allergic skin reaction H304 May be fatal if swallowed and enters airways H400 Very toxic to aquatic life H410 Very toxic to aquatic life with long lasting effects |  |  |
| Precautionary Statements |  |  |  |
|  | P210 Keep away from heat/sparks/open flames/hot surfaces. – No smoking P261 Avoid breathing dust/fume/gas/mist/vapour/spray P264 Wash thoroughly after handling P270 Do not eat, drink or smoke when using this product P271 Use only outdoors or in a well-ventilated area P273 Avoid release to the environment P280 Wear protective gloves/protective clothing/eye protection/face protection P301/310 IF SWALLOWED: Immediately call a POISON CENTRE or doctor/physician P301/312 IF SWALLOWED: call a POISON CENTRE or doctor/physician if you feel unwell P302/352 IF ON SKIN: Wash with plenty of soap and water P304/340 IF INHALED: Remove to fresh air and keep at rest in a position comfortable for breathing P305/351/358 IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses if present and easy to do. Continue rinsing P312 Call a POISON CENTRE or doctor/physician if you feel unwell. P330 Rinse mouth P331 Do not induce vomiting P333/313 If skin irritation or rash occurs: Get medical advice/attention P337/313 If eye irritation persists: Get medical advice/attention P391 Collect spillage. |  |  |
| Supplementary Precautionary Statements |  |  |  |
|  | None. |  |  |
| 2.3 |  | Other Hazards |  |
| PBT or vPvB according to Annex XIII |  |  | No additional data available. |
| Adverse physio-chemical properties |  |  | No additional data available. |
| Adverse effects on human health |  |  | No additional data available. |

##### 03. COMPOSITION/INFORMATION ON INGREDIENTS

| 3.2 | Mixtures |
| --- | --- |
|  | 25.0 - 50.0% Alpha-pinene CAS-No.: 80-56-8 EC No.: 201-291-9 Classification (EC 1272/2008) FL 3, ATO 5, SCI 2, SS 1B, AH 1 |
|  | 25.0 - 50.0% Beta-pinene CAS-No.: 127-91-3 EC No.: 204-872-5 Classification (EC 1272/2008) FL 3, ATO 5, SCI 2, SS 1B, AH 1 |
|  | 1.0 - 12.0% Limonene CAS-No.: 5989-27-5 EC No.: 227-813-5 Classification (EC 1272/2008) FL 3, SCI 2, SS 1B, AH 1, EH A1, EH C1 |
|  | 5.0 – 10.0% Phellandrenes CAS: 99-83-2, EINECS: 202-792-5 Classification (EC 1272/2008) FL 3, SCI 3, AH 1 |
|  | 1.0 - 5.0% Alpha-Terpineol CAS-No.: 98-55-5 EC No.: 202-680-6 Classification (EC 1272/2008) FL 4, ATO 5, SCI 2, EDI 2A, EH, A2 |
|  | 1.0 - 5.0% Terpinolene CAS-No.: 586-62-9 EC No.: 209-578-0 Classification (EC 1272/2008) FL 3, ATO 5, SCI 3, AH 1, EH A2, EH C2 |
|  | 0.1 - 1.0% Myrcene CAS-No.: 123-35-3 EC No.: 204-622-5 Classification (EC 1272/2008) FL 3, SCI 2, EDI 2A, AH 1 |
|  | 0.1 - 1.0% Camphene CAS-No.: 79-92-5 EC No.: 201-234-8 Classification (EC 1272/2008) SCI 3, EDI 2A, EH C1, EH A1, FS 2 |
|  | 0.1 - 1.0% Para-Cymene CAS-No.: 99-87-6 EC No.: 202-796-7 Classification (EC 1272/2008) FL 3, ATO 5, SCI 3, AH 1, EH A2, EH C2 |
|  | 0.1 - 1.0% Eugenol CAS-No.: 97-53-0 EC No.: 202-589-1 Classification (EC 1272/2008) ATO 5, SCI 3, EDI 2A, SS 1, EH A2 |
|  | 0.1 - 1.0% Methyl chavicol (Estragole) CAS-No.: 140-67-0 EC No.: 205-427-8 Classification (EC 1272/2008) FL 4, ATO 4, SCI 3, SS 1B, MUT 2, CAR 2 |
|  | 0.1 - 1.0% Terpinene-4-ol (4-Terpineol) CAS-No.: 562-74-3 EC No.: 209-235-5 Classification (EC 1272/2008) FL 4, ATO 4, ATD 5, SCI 2, EDI 2A, EH A3 |
|  | 0.1 - 1.0% Beta-Caryophyllene CAS-No.: 87-44-5 EC No.: 201-746-1 Classification (EC 1272/2008) SCI 3, AH 1 |
|  | Trace - 0.1% Eugenyl acetate CAS-No.: 93-28-7 EC No.: 202-235-6 Classification (EC 1272/2008) ATO 4, SCI 3 |

##### 04. FIRST AID MEASURES

| 4.1 | Description of first aid measures |  |
| --- | --- | --- |
| Inhalation |  | Remove casualty to fresh air, remove any contaminated clothing. Seek medical advice for any breathing difficulty. |
| Ingestion |  | Rinse mouth with water and give up to 0.2 litres of water to drink. Do NOT induce vomiting. Seek medical advice immediately. |
| Skin Contact |  | Wash off affected area with soap and running water. If irritation persists seek medical advice. |
| Eye Contact |  | Irrigate eye with plenty of water for 15 minutes with lids held open. Seek medical advice if irritation persists. |
| 4.2 | Most important symptoms and effects, both acute and delayed |  |
|  | No additional data available. |  |
| 4.3 | Indication of any immediate medical attention and special treatment needed |  |
|  | No additional data available. |  |

##### 05. FIRE-FIGHTING MEASURES

| 5.1 | Extinguishing Media |
| --- | --- |
|  | Fire can be extinguished using: Foam, carbon dioxide or dry powder. |
| 5.2 | Special hazards arising from the product |
|  | Smoke, Carbon Dioxide, Carbon Monoxide. |
| 5.3 | Advice for firefighters |
|  | Containers close to the fire should be removed or cooled with water. |

##### 06. ACCIDENTAL RELEASE MEASURES

| 6.1 | Personal precautions, protective equipment and emergency procedures |
| --- | --- |
|  | Use respirators if necessary and suitable protective clothing including goggles and gloves. |
| 6.2 | Environmental Precautions |
|  | Avoid contamination of drains. Dispose of in accordance with local regulations. In the event of any discharge into drains notify the relevant Authorities. |
| 6.3 | Methods and material for containment and cleaning up. |
|  | Gather spill in inert material, sand or earth. |
| 6.4 | Reference to other sections |
|  | See point 4, 8 & 13. |

##### 07. HANDLING AND STORAGE

| 7.1 | Precautions for safe handling |
| --- | --- |
|  | Use in a well ventilated area away from sources of ignition in accordance with good occupational hygiene and safety practices. NO SMOKING. |
| 7.2 | Conditions for safe storage, including any incompatibilities |
|  | Well filled tightly stoppered containers away from heat, light and ignition. Recommended storage at ambient temperature. |
| 7.3 | Specific end use(s) |
|  | No additional data available. |

##### 08. EXPOSURE CONTROLS/PERSONAL PROTECTION

| 8.1 | Control parameters |  |  |
| --- | --- | --- | --- |
|  |  | Not determined. |  |
| 8.2 | Exposure controls |  |  |
| Protective Equipment |  |  |  |
| Process Conditions |  |  | Provide eyewash station. |
| Engineering Measures |  |  | Provide adequate ventilation. |
| Respiratory Equipment |  |  | No specific recommendation made, but respiratory protection may still be required under exceptional circumstances when excessive air contamination exists. |
| Hand Protection |  |  | Safety gloves. |
| Eye Protection |  |  | Safety glasses or goggles. |
| Other Protection |  |  | Light protective clothing. |
| Hygiene Measures |  |  | Good housekeeping and work hygiene principles should be used when handling this material. |
| Personal Protection |  |  | Protective work clothing solvent resistant (should be checked regularly). |
| Skin Protection |  |  | Wear apron or protective clothing in case of splashes. |
| Environmental Exposure Controls |  |  | Avoid discharging into drainage water. Only eliminate by authorised companies. |

##### 09. PHYSICAL AND CHEMICAL PROPERTIES

| 9.1 | Information on basic physical and chemical properties |  |
| --- | --- | --- |
| Appearance |  | Clear liquid |
| Colour |  | Pale yellowish / yellow |
| Odour |  | Fresh rich balsamic woody piney |
| Relative Density |  | Approx. 0.865 @ 20°C |
| Flash Point (°C) |  | 35 |
| Refractive Index |  | Approx. 1.475 @ 20°C |
| Melting Point (°C) |  | No additional data available. |
| Boiling Point (°C) |  | No additional data available. |
| Vapour Pressure |  | No additional data available. |
| Solubility in Water @20°C |  | Insoluble in water. |
| Auto-ignition temperature (°C) |  | No additional data available. |
| 9.2 | Other information |  |
|  | No additional data available. |  |

##### 10. STABILITY AND REACTIVITY

| 10.1 |  | Reactivity |
| --- | --- | --- |
|  | Stable under normal conditions. |  |
| 10.2 |  | Chemical stability |
|  | Stable under normal temperature conditions. |  |
| 10.3 |  | Possible hazardous reactions |
|  | No additional data available. |  |
| 10.4 |  | Conditions to Avoid |
|  | Avoid heat, flames and other sources of ignition. |  |
| 10.5 |  | Incompatible materials |
|  | Avoid contact with strong oxidising materials. |  |
| 10.6 |  | Hazardous Decomposition Products |
|  | In case of fire, toxic gasses (CO, CO2, NOx) may be formed. |  |

##### 11. TOXOLOGICAL INFORMATION

| 11.1 |  | Information on toxicological effects |  |
| --- | --- | --- | --- |
|  | Acute Toxicity |  | No specific health warnings noted. |
|  | Skin corrosion / irritation |  | No specific health warnings noted. |
|  | Serious eye damage / irritation |  | No additional data available. |
|  | Respiratory or skin sensitisation |  | No specific health warnings noted. |
|  | Germ Cell Mutagenicity |  | No additional data available. |
|  | Carcinogenicity |  | No additional data available. |
|  | Reproductive toxicity |  | No additional data available. |
|  | STOT-single exposure |  | No additional data available. |
|  | STOT-repeated exposure |  | No additional data available. |
|  | Aspiration hazard |  | No additional data available. |
|  | Photo-toxicity |  | No additional data available. |
|  | Other Information |  | No additional data available. |

##### 12. ECOLOGICAL INFORMATION

| 12.1 |  | Toxicity |
| --- | --- | --- |
|  | Very toxic to aquatic life with long lasting effects. May cause long-term adverse effects in the aquatic environment. When spilled can contaminate soil, ground and surface water. |  |
| 12.2 |  | Persistence & degradability |
|  | No additional data available. |  |
| 12.3 |  | Bioaccumulation Potential |
|  | No additional data available. |  |
| 12.4 |  | Mobility in soil |
|  | No additional data available. |  |
| 12.5 |  | Results of PBT and vPvB Assessment |
|  | No additional data available. |  |
| 12.6 |  | Other adverse effects |
|  | No additional data available. |  |

##### 13. DISPOSAL CONSIDERATIONS

| 13.1 |  | Waste treatment methods |
| --- | --- | --- |
|  | Dispose of according to the regulations in your area. Do not discard into drainage systems or elsewhere into the environment. |  |

##### 14. TRANSPORT INFORMATION

| 14.1 |  | UN number |  |
| --- | --- | --- | --- |
|  |  | UN No. Road | 1169 |
|  |  | UN No. SEA | 1169 |
|  |  | UN No. AIR | 1169 |
| 14.2 |  | UN proper shipping name |  |
|  | Extracts, aromatic, liquid |  |  |
| 14.3 |  | Transport hazard class(es) |  |
|  | ADR/RID/ADN – Class 3 ADR/RID/ADN – Class 3 IMDG – Class 3 ICAO – Class 3 Transport Labels TUNNEL RESTRICTION CODE (D/E) HAZCHEM CODE •3YE EMS F-E, S-D |  |  |
| 14.4 |  | Packing group |  |
|  | ADR/RID/ADN Packing group – III IMDG Packing group – III ICAO Packing group – III |  |  |
| 14.5 |  | Environmental hazards |  |
|  | Environmentally Hazardous Substance/Marine Pollutant |  |  |
| 14.6 |  | Special precautions for user |  |
|  | None |  |  |
| 14.7 |  | Transport in bulk according to Annex II of MARPOL73/78 and the IBC code |  |
|  | - |  |  |

##### 15. REGULATORY INFORMATION

| 15.1 |  | Safety, health and environmental regulations/legislation specific for the substance or mixture |
| --- | --- | --- |
|  | Statutory Instruments The Chemicals (Hazard Information and Packaging for Supply) Regulations 2009 (S.I 2009 No. 716). Guidance Notes Workplace Exposure Limits EH40. CHIP for everyone HSG(108). EU Legislation Regulation (EC) No 1907/2006 of the European Parliament and of the Council of 18 December 2006 concerning the Registration, Evaluation, Authorisation and Restriction of Chemicals (REACH), establishing a European Chemicals Agency, amending Directive 1999/45/EC and repealing Council Regulation (EEC) No 793/93 and Commission Regulation (EC) No 1488/94 as well as Council Directive 76/769/EEC and Commission Directives 91/155/EEC, 93/67/EEC, 93/105/EC and 2000/21/EC, including amendments. |  |
| 15.2 |  | Chemical safety assessment |
|  | No additional information available. |  |

##### 16. OTHER INFORMATION

| Hazard and/or Precautionary Statements in Full | H226 Flammable liquid and vapour H302 Harmful if swallowed H312 Harmful in contact with skin H332 Harmful if inhaled H315 Causes skin irritation H319 Causes serious eye irritation H317 May cause an allergic skin reaction H304 May be fatal if swallowed and enters airways H400 Very toxic to aquatic life H410 Very toxic to aquatic life with long lasting effects. |
| --- | --- |
| Other Information | None |
| Revision Date | 08/09/15 |
| Reason for revision | New SDS |
| Rev No/Repl, SDS Generated | 01 |

## DISCLAIMER: This information relates only to the specific material designated and may not be valid for such material used in combination with any other materials or in any process. Such information is, to the best of the company's knowledge and belief, accurate and reliable as of the date indicated. However, no warranty guarantee or representation is made to its accuracy, reliability or completeness. It is the user's responsibility to satisfy himself as to the suitability of such information for his own particular use.