Tension Screen

User Manual

Product Electrical specification

Voltage: 100V~120V 60Hz  220V~240V 50Hz

Power: 130W (MAX)

Method of installation

lnstall the screen in the location where the audience

can see the whole screen when it is fully extended.

The screen can be wall mounted or ceiling mounted,

users may choose the mounting method according to

their needs. Whichever method of mounting is used,

ensure the right equipment is used. For mounting on

the wall or on wooden surface ceiling, use ø5x35 mm

Screws.

Ceiling Hanging:

Unscrew the locking screws

on both sliding pieces.

Move the sliding pieces to desired position. Tighten the holding ring on both sliding pieces until the sliding pieces do not slide freely on the housing.

The “J” hook should in installed at least 50mm from the wall

There is a 10mm clearance between the wall and the screen housing after installation

Wall Mounting:

For mounting on a brick, concrete or plaster surface use

ø5x35mm screws with screw anchors or wall plugs.

Mark 2 horizontal parallel

lines on wall at desired

height. Drill holes on wall

According to the openings

on wall mount bracket

Insert the provided plastic                                              anchor into the holes.

The fasten the wall mount                                               brackets on wall.

Hang the screen onto the

wall mount bracket as

shown on diagram

Afterwards tighten the

securing screw, Prevention

the screw comes out from

wall mount bracket.

Screen operation

Wireless control unit operation

To lower the projection surface

press the bottom button on the control unit, the projection surface will lower

itself, when it is fully extended it will stop automatically.

To retract the projection surface

press the top button on the control unit, the projection surface will retract

into its case, when it is fully retracted it will stop automatically.

To stop the projection surface

Press the middle button on the control unit. For detail remote control set-up, please refer to the individual remote Control Manual.