| SAFETY DATA SHEET according to Regulation (EC) No. 1907/2006 |  |
| --- | --- |
| GREEN TEA & PATCHOULI |  |
| Version 2 Revision Date 19.09.2018 | Print Date 20.12.2018 |
| SECTION 1: IDENTIFICATION OF THE SUBSTANCE/MIXTURE AND OF THE COMPANY/UNDERTAKING - Product identifier Commercial Product Name : GREEN TEA & PATCHOULI - Relevant identified uses of the substance or mixture and uses advised against Use of the Substance/Mixture : Fragrance mix - Details of the supplier of the safety data sheet Candle Supply Pty Ltd Unit 3 8-9 Lagana Place Wetherill Park, NSW 2164 ABN: 70612899626 Phone Number: 02 8741 4000 e-mail: customerservice@candlesupply.com.au - Emergency telephone number In an emergency call: 13 11 26 |  |
| SECTION 2: HAZARDS IDENTIFICATION |  |
| - Classification of the substance or mixture |  |
| Classification (REGULATION (EC) No 1272/2008) |  |
| Skin sensitisation, Category 1 | H317 |
| Chronic aquatic toxicity, Category 3 | H412 |
| 2.2 Label elements |  |
| Labelling (REGULATION (EC) No 1272/2008) |  |
| Hazard pictograms : |  |
| Signal word : Warning |  |
| 1/9 |  |

| Chemical name | CAS-No. EC-No. Registration number | Classification (REGULATION (EC) No 1272/2008) | Concentration [%] |
| --- | --- | --- | --- |
| Patchouli, ext. | 84238-39-1 8014-09-3 282-493-4 | Asp. Tox.: 1; H304 Aquatic Chronic: 2; H411 | >= 1 - < 5 |
| Linalool | 78-70-6 201-134-4 | Skin Irrit.: 2; H315 Eye Irrit.: 2; H319 Skin Sens.: 1B; H317 | >= 1 - < 5 |
| (R)-p-Mentha-1,8-diene | 5989-27-5 227-813-5 | Asp. Tox.: 1; H304 Aquatic Chronic: 1; H410 Aquatic Acute: 1; H400 Skin Sens.: 1B; H317 Skin Irrit.: 2; H315 Flam. Liq.: 3; H226 | >= 1 - < 5 |
| Allyl hexanoate 1) | 123-68-2 204-642-4 01-2119983573-26-0000 | Acute Tox.: 3 (Inhalation); H331 Acute Tox.: 3 (Oral); H301 Acute Tox.: 3 (Dermal); H311 Aquatic Acute: 1; H400 AquaticChronic: 3; H412 | >= 0,1 - < 1 |

SAFETY DATA SHEET according to Regulation (EC) No. 1907/2006

GREEN TEA & PATCHOULI

Version 2 Revision Date 19.09.2018 Print Date 20.12.2018

Hazard statements : H317 May cause an allergic skin reaction. H412 Harmful to aquatic life with long lasting effects. Precautionary statements : Prevention: P272 Contaminated work clothing should not be allowed out of the workplace. P273 Avoid release to the environment. P280 Wear protective gloves/protective clothing/eye protection/face protection. Response: P302 + P352 IF ON SKIN: Wash with plenty of water. P333 + P313 If skin irritation or rash occurs: Get medical advice/ attention. P362 + P364 Take off contaminated clothing and wash it before reuse. Contains : CYCLOHEXENE, 1-METHYL-4-(1-METHYLETHENYL)-, (R)-, LINALOOL. 2.3 Other hazards No data available

SECTION 3: COMPOSITION/INFORMATION ON INGREDIENTS - Substances No data available - Mixtures Hazardous components

2/9

SAFETY DATA SHEET according to Regulation (EC) No. 1907/2006

GREEN TEA & PATCHOULI

Version 2 Revision Date 19.09.2018 Print Date 20.12.2018

Linalyl acetate 115-95-7 Skin Irrit.: 2; H315 >= 0,1 - < 1 204-116-4 Eye Irrit.: 2; H319 Skin Sens.: 1B; H317 For the full text of the H-Statements mentioned in this Section, see Section 16. Acute aquatic toxicity: 1) 0,1 < L(E)C50 <= 1 mg/l

SECTION 4: FIRST AID MEASURES - Description of first aid measures General advice : Move out of dangerous area. Never give anything by mouth to an unconscious person. Inhalation : Move to fresh air. Skin contact : Take off all contaminated clothing immediately. If skin irritation persists, call a physician. Wash off immediately with plenty of water for at least 15 minutes. Eye contact : Irrigate copiously with clean, fresh water for at least 10 minutes, holding the eyelids apart. Remove contact lenses. Ingestion : Clean mouth with water and drink afterwards plenty of water. Prevent vomiting if possible. If a person vomits when lying on his back, place him in the recovery position. Never give anything by mouth to an unconscious person. - Most important symptoms and effects, both acute and delayed No data available - Indication of any immediate medical attention and special treatment needed No data available

SECTION 5: FIREFIGHTING MEASURES - Extinguishing media Suitable extinguishing media : Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide. Extinguishing media which : High volume water jet shall not be used for safety reasons - Special hazards arising from the substance or mixture Specific hazards during : Do not use a solid water stream as it may scatter and spread fire. firefighting Do not allow run-off from fire fighting to enter drains or water courses. - Advice for firefighters Special protective equipment : In the event of fire, wear self-contained breathing apparatus. for firefighters

3/9

SAFETY DATA SHEET according to Regulation (EC) No. 1907/2006

GREEN TEA & PATCHOULI

Version 2 Revision Date 19.09.2018 Print Date 20.12.2018

Further information : In the event of fire and/or explosion do not breathe fumes. Use water spray to cool unopened containers. Fire residues and contaminated fire extinguishing water must be disposed of in accordance with local regulations. Collect contaminated fire extinguishing water separately. This must not be discharged into drains.

SECTION 6: ACCIDENTAL RELEASE MEASURES - Personal precautions, protective equipment and emergency procedures Personal precautions : Ensure adequate ventilation. Use personal protective equipment. - Environmental precautions Environmental precautions : Should not be released into the environment. Avoid subsoil penetration. Prevent further leakage or spillage if safe to do so. If the product contaminates rivers and lakes or drains inform respective authorities. - Methods and materials for containment and cleaning up Methods for cleaning up : Soak up with inert absorbent material (e.g. sand, silica gel, acid binder, universal binder, sawdust). Dam up. Wipe up with absorbent material (e.g. cloth, fleece). Additional advice : Local authorities should be advised if significant spillages cannot be contained. Suppress (knock down) gases/vapours/mists with a water spray jet. - Reference to other sections For personal protection see section 8.

SECTION 7: HANDLING AND STORAGE - Precautions for safe handling Advice on safe handling : Dispose of rinse water in accordance with local and national regulations. For personal protection see section 8. Advice on protection against : When using do not smoke. fire and explosion - Conditions for safe storage, including any incompatibilities Requirements for storage : Store in original container. areas and containers Keep container tightly closed in a dry and well-ventilated place. To maintain product quality, do not store in heat or direct sunlight. Containers which are opened must be carefully resealed and kept upright to prevent leakage. Advice on common storage : No special restrictions on storage with other products.

4/9

| Components | CAS-No. | Control parameters | Update | Basis |
| --- | --- | --- | --- | --- |
| (R)-p-Mentha-1,8-diene | 5989-27-5 | AGW 5 ppm | 04 2013 | TRGS 900 |

SAFETY DATA SHEET according to Regulation (EC) No. 1907/2006

GREEN TEA & PATCHOULI

Version 2 Revision Date 19.09.2018 Print Date 20.12.2018

Other data : No decomposition if stored and applied as directed. 7.3 Specific end use(s) Fragrance mix

SECTION 8: EXPOSURE CONTROLS/PERSONAL PROTECTION - Control parameters Components with workplace control parameters, Germany: - Exposure controls Engineering measures none Personal protective equipment Respiratory protection : No personal respiratory protective equipment normally required. Hand protection : Wear chemicals-resistant gloves, e.g. safety gloves of chloroprene Level 2 or of butyl rubber Level 6. Eye protection : Safety glasses Skin and body protection : Lightweight protective clothing Hygiene measures : Wash hands before breaks and immediately after handling the product. Protective measures : When using, do not eat, drink or smoke.

SECTION 9: PHYSICAL AND CHEMICAL PROPERTIES 9.1 Information on basic physical and chemical properties Appearance

Form : liquid Colour : light yellow to yellow-brown

Odour : characteristic Odour Threshold : No data available Safety data Flash point : 97 °C Water solubility : immiscible

5/9

|  | SAFETY DATA SHEET according to Regulation (EC) No. 1907/2006 |  |  |
| --- | --- | --- | --- |
|  | GREEN TEA & PATCHOULI |  |  |
|  | Version 2 Revision Date 19.09.2018 Print Date 20.12.2018 |  |  |
|  | Relative density | : | 0,9069 - 0,9269 at 20 °C relation to density of water at 4°C |
|  | Bulk density : Relative vapour density : not determined Vapour pressure : < 1 kPa at 50 °C calculated Melting point/freezing point : not determined Boiling point : not determined Partition coefficient: n- : log Pow: octanol/water Not applicable pH : Not applicable Evaporation rate : Not applicable Flammability (solid, gas) : Not applicable Auto-ignition temperature : The substance or mixture is not classified as self heating. Decomposition temperature : No data available Oxidizing properties : The substance or mixture is not classified as oxidizing. Explosive properties : Due to its structural properties, the product is not classified as explosive Lower explosion limit : Vapours may form explosive mixtures with air. Upper explosion limit : Vapours may form explosive mixtures with air. A final assessment of potential explosion hazards can only be made on a case-by-case basis when precise information is available on product handling, production setup and the circumstances surrounding the use of the product. 9.2 Other data None. |  |  |
|  | SECTION 10: STABILITY AND REACTIVITY - Reactivity : No data available - Chemical stability : No data available - Possibility of hazardous : No dangerous reaction known under conditions of normal use. reactions - Conditions to avoid : No data available - Incompatible materials : No data available to avoid - Hazardous decomposition : No data available products |  |  |
|  | 6/9 |  |  |

| SAFETY DATA SHEET according to Regulation (EC) No. 1907/2006 |  |  |
| --- | --- | --- |
|  | GREEN TEA & PATCHOULI |  |
| Version 2 | Revision Date 19.09.2018 | Print Date 20.12.2018 |
| SECTION 11: TOXICOLOGICAL INFORMATION |  |  |
| 11.1 Information on toxicological effects |  |  |
| Acute oral toxicity : Acute toxicity estimate : > 2.000 mg/kg, Calculation method |  |  |
| Acute inhalation toxicity : Acute toxicity estimate : > 20 mg/l , 4 h, Calculation method |  |  |
| Acute dermal toxicity : Acute toxicity estimate : > 2.000 mg/kg, Calculation method |  |  |
| Repeated dose toxicity : No data available |  |  |
| Acute toxicity (other routes : No data available |  |  |
| of administration) |  |  |
| Skin irritation : No data available |  |  |
| Eye irritation : No data available |  |  |
| Sensitisation : No data available |  |  |
| Mutagenicity : No data available |  |  |
| Carcinogenicity : No data available |  |  |
| Reproductive toxicity : No data available |  |  |
| Teratogenicity : No data available |  |  |
| Specific target organ toxicity : No data available |  |  |
| - single exposure |  |  |
| Specific target organ toxicity : No data available |  |  |
| - repeated exposure |  |  |
| Aspiration toxicity : No data available |  |  |
| SECTION 12: ECOLOGICAL INFORMATION 12.1 Toxicity No data available |  |  |
| 7/9 |  |  |

SAFETY DATA SHEET according to Regulation (EC) No. 1907/2006

GREEN TEA & PATCHOULI

Version 2 Revision Date 19.09.2018 Print Date 20.12.2018

- Persistence and degradability No data available - Bioaccumulative potential No data available - Mobility in soil No data available - Results of PBT and vPvB assessment This substance/mixture contains no components considered to be either persistent, bioaccumulative and toxic (PBT), or very persistent and very bioaccumulative (vPvB) at levels of 0.1% or higher. - Other adverse effects Harmful to aquatic life with long lasting effects.

SECTION 13: DISPOSAL CONSIDERATIONS - Waste treatment methods Product : Where possible recycling is preferred to disposal or incineration. If recycling is not practicable, dispose of in compliance with local regulations. Contaminated packaging : Empty remaining contents. Dispose of as unused product.

SECTION 14: TRANSPORT INFORMATION - UN number Not classified as dangerous in the meaning of transport regulations. - Proper shipping name Not classified as dangerous in the meaning of transport regulations. - Transport hazard class Not classified as dangerous in the meaning of transport regulations. - Packing group Not classified as dangerous in the meaning of transport regulations. - Environmental hazards Not classified as dangerous in the meaning of transport regulations. - Special precautions for user Not classified as dangerous in the meaning of transport regulations. - Transport in bulk according to Annex II of MARPOL 73/78 and the IBC Code Not classified as dangerous in the meaning of transport regulations.

8/9

| SAFETY DATA SHEET according to Regulation (EC) No. 1907/2006 |  |  |
| --- | --- | --- |
|  | GREEN TEA & PATCHOULI |  |
| Version 2 | Revision Date 19.09.2018 | Print Date 20.12.2018 |
| SECTION 15: REGULATORY INFORMATION |  |  |
| 15.1 Safety, health and environmental regulations/legislation specific for the substance or mixture |  |  |
| National legislation |  |  |
| Risk classification according : Flash Point > 55 °C up to 100 °C, at 15 °C not miscible with water |  |  |
| to BetrSichV (Germany) |  |  |
| Water contaminating class : WGK 2 (water endangering) |  |  |
| (Germany) Calculation acc. Annex 4 to VwVwS |  |  |
| 15.2 Chemical safety assessment |  |  |
| No data available |  |  |
| SECTION 16: OTHER INFORMATION |  |  |
| Full text of H-Statements referred to under sections 2 and 3.: |  |  |
| H226 Flammable liquid and vapour. |  |  |
| H301 Toxic if swallowed. |  |  |
| H304 May be fatal if swallowed and enters airways. |  |  |
| H311 Toxic in contact with skin. |  |  |
| H315 Causes skin irritation. |  |  |
| H317 May cause an allergic skin reaction. |  |  |
| H319 Causes serious eye irritation. |  |  |
| H331 Toxic if inhaled. |  |  |
| H400 Very toxic to aquatic life. |  |  |
| H410 Very toxic to aquatic life with long lasting effects. |  |  |
| H411 Toxic to aquatic life with long lasting effects. |  |  |
| H412 Harmful to aquatic life with long lasting effects. |  |  |
| Further information |  |  |
| The data contained in this Safety Data Sheet is accurate to the best knowledge of Candle Supply Pty Ltd, applies to the product as supplied Candle Supply Pty Ltd, and does not relate to use in combination with any other material or in any process. Data and information is furnished without warranty expressed or implied, nor does Candle Supply Pty Ltd, assume responsibility for use or reliance upon this data. This SDS is current to the date listed above. However, the GHS classifications may change due to hazard communication updates by the overseeing governing body. For the most current SDS information please contact customerservice@candlesupply.com.au |  |  |
| 9/9 |  |  |