Dispense system shutdown procedure

The following is intended as a guide for those steps that are necessary to safely close down the dispense system and related equipment in pubs and bars in readiness for the time when trade will resume.

Closedown should take into account the safety of the site, the hygienic operation of the dispense system and the security of any stock and assets, including arranging for their return by the owning brewer.

Dispense system (Keg & Cask)

To close your system down:

- All connected kegs and casks should be disconnected.

- Empty and clean all lines following standard procedures and ensure that fob detectors/cellar buoys are set and left in the clean position

- Once cleaned all dispense lines should be blown through

Blowing though procedure:

•      Ensure the cleaning bottle is empty and line cleaning feed tube is

inside the bottle

•      At the bar open each tap until air is coming through every line

•      Turn off cleaning gas and any assist pumps

•      Turn off remote coolers at the plug

•      Turn gas off at individual valves and main supply

For cask lines once cleaned pull through so only air is coming from each line

It is important that dispense lines are not left charged with water or cleaning fluid as this can cause irrevocable damage to the system.

- All connected equipment i.e. keg couplers, cask taps, filters & tap nozzles should be cleaned with an appropriate sanitiser but should not be left to soak

- Tap nozzles should be reconnected after sanitizing

- Following cleaning couplers & taps should not be left in contact with the floor

- After cleaning dispense gasses should be turned off at both the primary (wheel style valves turned clockwise to close) and secondary valves. It is not necessary to disconnect gas bottles from the system.

- Mop up any spilt beer

Dispense/Cellar/Bar Equipment

- all electrical equipment i.e. remote coolers, dish/glasswashers & ice makers etc must be switched off, turned off at socket & unplugged:

- Dish/ glasswashers should be cleaned, sanitised and drained including filters and spray arms and doors left open to avoid mould developing

- Ice makers should be emptied, and any ice allowed to melt completely before being thoroughly cleaned and sanitized using standard procedures. Any removable machine components i.e. ice bins should be cleaned separately & thoroughly. Any access doors

should be left open

- If you have any unbroached kegs in the cellar, then we would advise to leave cellar cooling on so they will be kept in the correct environment to use when ready to re-open.

- If cellar is empty, then fine to turn cellar cooling off

*Credits to the BBPA & Avani Solutions