## Identification

- Product identifier

Trade name:  MARK3 MAX BITE

- Application of the substance or the preparation

Addition curing vinyl polysiloxane dental impression materials (consisting of Base & Catalyst)

- Manufacturer/Supplier

Cargus International, Inc.

135 North Route 9W

Congers, New York 10920

Tel: (845) 267-2600

E-mail: info@cargus.com

- 24H Emergency Telephone numbers

ChemTel Contract No. MIS0007996

North America: 1-800-255-3924

International: +01-813-248-0585

## Hazards Identifications

The preparation of this document has been carried out according to the U.S. OSHA Hazard Communication Standard. Therefore, all the known hazards of the product or components have to be included regardless the potential risk.

- Hazard classification

Skin sensitizer: Category 1

- Label elements

Signal word: Warning.

Symbols: Exclamation mark.

Pictograms

- Hazard statement

May cause an allergic skin reaction.

Causes severe eye irritation.

- Precautionary statements

Prevention:

Wear protective gloves, protective clothing and eye protection.

Work clothing which is contaminated must not be allowed out of workplace.

Wash thoroughly after handling.

Response:

IF ON SKIN: Wash with plenty of water and soap. In case skin irritation occurs, seek medical attention. Contaminated clothing must be washed before reuse.

IF IN EYES: Rinse with caution with plenty of water for several minutes. In case contact lenses are worn remove and continue rinsing. If you feel unwell get medical attention.

Disposal: Dispose of contents/container in accordance with applicable local/regional/national/ international regulations.

- Classification system

NFPA ratings (scale 0 – 4)

Health = 2

Fire = 0

Reactivity = 0

HMIS ratings (scale 0 – 4)

Health = 2

Fire = 0

Reactivity = 0

- Hazards not otherwise classified

None.

## Composition/Information on ingredients

- Chemical characterization

Mixture of addition-curing silicones.

| C.A.S. No. | Component | % Wt |
| --- | --- | --- |
| 68083-19-2 | Polydimethylsiloxane vinyl terminated | 20 – 40 |
| 68037-59-2 | Dimethylsiloxane copolymer | 1 – 10 |
| 14808-60-7 | Quartz silica | 30 – 50 |
| 27306-78-1 | Polysiloxane | < 5 |

## First aid measures

Description of first aid measures

- After inhalation

Remove person to fresh air. Get medical attention if you feel unwell.

- After contact with skin

Immediately flush with water and then wash with soap and water. In case skin irritation persists, get medical attention.

- After contact with eyes

Rinse with copious amounts of water for at least 15 minutes. Get medical attention.

- After ingestion

Flush mouth with water and get medical attention.

- Further information

None

## Fire-fighting measures

- Suitable extinguishing media

Water spray, foam, carbon dioxide, dry chemicals depending on the materials involved in the fire.

- Extinguishing media to avoid

Not known.

- Special hazards arising from combustion

No relevant information is available for this product.

- Special protective equipment for fire-fighters

No special measures are required.

## Accidental release measures

- Personal precautions, protective equipment and emergency procedures

Ventilate area. Avoid contact with skin and eyes.

- Environmental precautions

Do not allow to enter sewers or ground water

- Methods for cleaning up/collecting

Collect mechanically. The product may be absorbed by liquid-binding material (sand, diatomite, acid binders).

## Handling and storage

- Precautions for safe handling

Adequately trained personnel should handle this product.  Do not eat, drink or smoke when using the product. Wash thoroughly after handling. Contaminated clothing should not be allowed out of the workplace and in any case it should be washed before reuse. Keep container tightly closed.

- Precautions in case of fire and explosion

Extinguish all sources of ignition. No special measures required.

- Conditions for safe storage, including any incompatibilities

Store product in a cool and dry place. Avoid direct sunlight, strong bases, acids, oxidizing agents and amines. Store in the original container.

## Exposure controls/personal protection

- Control parameters

Occupational exposure limits

In case a component is shown in section 3 but it does not appear in the table below, there are not any available data for this component.

| C.A.S. No. | Component | Agency | Limit |
| --- | --- | --- | --- |
| 14808-60-7 | Quartz silica | ACGIH | TWA: 0.025* mg/m3 *respirable fraction |
| 14808-60-7 | Quartz silica | OSHA | TWA: 0.3* mg/m3 *concentration as a total of dust TWA: 0.1* mg/m3 (2.4 millions of particles/cu. ft.) *concentration respirable |

ACGIH: American Conference of Government Industrial Hygienists

OSHA: United States Department of Labour – Occupational Safety and Health Administration

TWA: Time-Weighted-Average

- Exposure control

Personal protective equipment (PPE)

Ventilation: Ensure adequate ventilation of the premises where the product is stored and/or handled.

Respiratory protection:  Not required.

Hand protection: Suitable protective gloves may be worn. The use of vinyl gloves is suggested.

(Setting time of polyvinylsiloxanes is inhibited by latex gloves).

Eye protection: Safety goggles are recommended.

Skin protection: Do not eat, drink or smoke when using the product.

Other: Lab coat is suggested when using the product.

## Physical and chemical properties

- Information on basic physical and chemical properties

Appearance: 		Bicomponent pastes/viscous liquids

Colour: 			Different

Odour: 			Characteristic

Odour threshold:		Not available

pH value		Not applicable

Melting point:		Not applicable

Boiling point:		Not applicable

Flash point:		Not applicable

Evaporation rate:		No data available

Flammability (solid, gas):	Not applicable

Density:			1,2-1,8 g/cm3

Vapour pressure:		Not applicable

Vapour density:		Not applicable

Viscosity:		Not determined

Solubility:		Insoluble in water

Auto ignition:		Product is not self igniting

Flash point:		No flash point

Boiling point:		Not applicable

Viscosity:		No data available

## Stability and reactivity

- Reactivity

No relevant information available.

- Chemical stability

Stable under normal storage and handle conditions.

- Hazardous decomposition products

None, if used in accordance with instructions.

- Conditions to avoid

No relevant information available, if used in accordance with instructions.

- Possibility of hazardous reactions

None, if used in accordance with instructions

## Toxicological information

It is required by the U.S. OSHA Hazard Communication Standard to include all the known hazards of the product or components included regardless the potential risk.

All the information presented below corresponds to the uncured components included in the product. Once the product is mixed according to the instructions for use it is safe for its intended use.

- Information on toxicological effects

Inhalation: No adverse health effects are expected.

Skin contact: May cause mild irritation.

Eye contact: May cause irritation.

Ingestion: There is a possibility of gastrointestinal irritation with the symptoms being stomach pain and upset, nausea, diarrhoea and vomiting.

- Additional toxicological information

Carcinogenicity:

During normal / intended use it is not expected to cause the following health effect(s) during exposure

| C.A.S. No. | Component | Class description | Regulation |
| --- | --- | --- | --- |
| 14808-60-7 | Quartz silica | Group 1: Carcinogenic to humans | International Agency for Research on Cancer |

Toxicological data:

In case a component is shown in section 3 but it does not appear in the table below, there are not any available data for this component.

Acute toxicity

| Component | Route | Species | Value |
| --- | --- | --- | --- |
| Polydimethylsiloxane vinyl terminated | Dermal | Rabbit | LD50 > 15,440 mg/Kg |
|  | Ingestion | Rat | LD50 > 15,440 mg/Kg |
| Dimethylsiloxane copolymer | Dermal | Rabbit | LD50 > 2,000 mg/Kg |
|  | Ingestion | Rat | LD50 > 2,000 mg/Kg |
|  | Inhalation | Rat | LC50 4.2 mg/l |
| Quartz silica | Dermal |  | LD50 > 5,000 mg/Kg |
|  | Ingestion |  | LD50 > 5,000 mg/Kg |
| Polysiloxane | Dermal | Rabbit | LD50 > 2,000 mg/Kg |
|  | Ingestion | Rat | LD50 > 2,000 mg/Kg |
|  | Inhalation | Rat | LC50 2 mg/l |

Skin corrosion/irritation

| Component | Species | Value |
| --- | --- | --- |
| Polydimethylsiloxane vinyl terminated | Rabbit | Mild irritation |
| Quartz silica |  | No significant irritation |

| Polysiloxane | Rabbit | No significant irritation |
| --- | --- | --- |

Serious eye damage/Irritation

| Component | Species | Value |
| --- | --- | --- |
| Polydimethylsiloxane vinyl terminated | Rabbit | Mild irritation |
| Polysiloxane | Rabbit | Severe irritation |

Skin sensitization

| Component | Species | Value |
| --- | --- | --- |
| Polysiloxane | Guinea pig | Not sensitizing |

Respiratory sensitization:

In case a component is shown in section 3 but it does not appear in the table below, there are not any available data for this component.

Germ cell mutagenicity

| Component | Route | Value |
| --- | --- | --- |
| Polysiloxane | In vitro | Not mutagenic |
| Quartz silica | In vitro | Not mutagenic |

Carcinogenicity

| Component | Route | Species | Value |
| --- | --- | --- | --- |
| Quartz silica | Inhalation | Human and Animal | Carcinogenic |

Reproduced toxicity:

In case a component is shown in section 3 but it does not appear in the table below, there are not any available data for this component.

Reproduced and/or development effects

| Component | Route | Value | Species | Test Result | Exposure duration |
| --- | --- | --- | --- | --- | --- |
| Polysiloxane | Ingestion | There is evidence for positive reproduction but limited data for classification | Rat | NOAEL 450 mg/Kg/day | pre-mating & during gestation |

Target organ(s):

In case a component is shown in section 3 but it does not appear in the table below, there are not any available data for this component.

Specific target organ toxicity – single exposure

In case a component is shown in section 3 but it does not appear here, there are not any available data for this component

Specific target organ toxicity – repeated exposure

| Component | Route | Target organ | Value | Species | Test Result | Exposure duration |
| --- | --- | --- | --- | --- | --- | --- |
| Quartz silica | Inhalation | silicosis | Causes damage to organs through prolonged or repeated exposure | Human | NOAEL Not available | Occupational exposure |

Aspiration hazard:

In case a component is shown in section 3 but it does not appear here, there are not any available data for this component.

## Ecological information

If material is properly used and handled no ecological problems are to be anticipated

Do not discard material into drains/sewers or aquatic environment

## Disposal considerations

Disposal must be carried out in accordance to the local and national regulations currently in force

## Transport information

| UN-Number DOT, ADR, ADN, IMDG, IATA | None |
| --- | --- |
| UN proper shipping name DOT, ADR, ADN, IMDG, IATA | None |
| Transport hazard class DOT, ADR, ADN, IMDG, IATA Class | None |
| Packing group DOT, ADR, IMDG, IATA | None |

## Regulatory information

The product is not considered hazardous under D. Lgs. 65/03.

The product is a medical device according to EC directive 93/42/EEC

This product is classified as a medical device under US regulations and it has been reviewed by the US Food and Drug Administration (FDA)

## Other information

CAUTION: PRODUCT FOR PROFESSIONAL USE ONLY

The information provided above is based on our present knowledge and experience. This safety data sheet refers explicitly to the product indicated and comprises no guarantee of particular quality. Any use of this product in any way not indicated on this safety data sheet will be exclusively under the user’s responsibility