TURNER BRAKE

DELUXE KIT REAR DISC PACKING LIST

1     LEFT REAR CALIPER MOUNTING BRACKET

1      RIGHT REAR CALIPER MOUNTING BRACKET

6      3/8 x 1 ½   GRADE 8 HEX HEAD BOLTS *

2      3/8 x 1 ¼   GRADE 8 HEX HEAD BOLTS*

8      3/8   LOCK WASHERS

8      3/8   GRADE 8 LOCKNUTS

2       AXLE BEARING RETAINERS

- BRAKE HOSE’S WITH WASHERS

- BANJO BOLTS

- LEFT LOADED REAR CALIPER

- RIGHT LOADED CALIPER

- LEFT ROTOR

- RIGHT ROTOR

10       NEW WHEEL STUDS (Not Used with Flanged Axles)

4        WIRE TIES

1        SET COMPLETE INSTRUCTIONS

*NOTE, EVEN THOUGH LOCK WASHERS ARE PROVIDED IN THIS

BRAKE KIT, WE HIGHLY RECOMMEND THE USE OF RED LOCTITE

1

## TURNER BRAKE

## STUDEBAKER REAR DISC BRAKE KIT

INSTALLATION INSTRUCTIONS

WHEN ALL ELSE FAILS;

READ THESE INSTRUCTIONS!

Remove the stock rear brakes, as per the Studebaker shop manual; you will also need to remove the brake backing plates. NOW is a good time to re-pack the rear wheel bearings, and set the rear axle endplay, AS per the Studebaker shop manual.

Once the rear bearings are re-packed and the end play adjusted, you are ready to install the TURNER BRAKE rear disc brake brackets. These brackets are stamped LEFT & RIGHT, The left side is the driver’s side, and the right side is the passenger side!

Using the 3, 3/8” x 1 ½” grade 8 bolts, and 1, 1 ¼” grade 8 bolt, assemble the bracket to the inside of the rear axle flange, so the caliper will be up and behind the axle. Then install the round bearing retainer, the stock seal, lock washers, lock nuts, and tighten these bolts to 55 ft. lbs of torque.

Next you will need to remove the brake drum from the stock hubs. Being careful not to damage the hub or drum, Even though you are not going to use the drums, they still have some value!! This may be something you may want to have a machine shop do for you, as this can damage both the drum and hub!

DO NOT THROW THEM AWAY!

Once you have the hubs separated from the drums, press the new studs into your existing hubs, Re-install the hubs and torque them as per the Studebaker shop manual. The rotors that are supplied are LEFT & RIGHT.  Next install the Left caliper on the left side and Right caliper on the right side, making sure the emergency brake cable brackets are at the top, along with the bleeder screw.

No you are ready to install the emergency brake cable!

Remove the stock emergency brake cable, as per the Studebaker shop manual, and install the emergency brake cable.* Once the cable is loosely in place, before attaching the front of the cable, it is much easier to attach the cable to the calipers. Once that is done, finish attaching the cable to the emergency brake handle.

Adjustment of the TURNER BRAKE rear disc brake emergency brake cable is accomplished using the original Studebaker adjustment. Take up the slack in the rear cable by tightening the front parking brake cable adjustment nut. PULL or step on the emergency brake THREE times and RELEASE, check at the rear calipers, look at the emergency brake lever, be sure that it is back against the stop when released, if it is, Then check the position of the lever when the emergency brake is engaged, it should be pulled forward and you should not be able to turn the wheels.

*For all applications, please see the enclosed paperwork.

2

TEST FOR PROPER OPERATION OF THE EMERGENCY BRAKE, YOU NEVER KNOW WHEN YOUR LIFE MAY DEPEND ON IT!

To keep the emergency brake adjusted and in proper working condition on your

TURNER BRAKE rear disc conversion,

ALWAYS SET YOUR EMERGENCY BRAKE WHEN PARKING!

Now it is time to install the rear brake hoses, Install the 90 degree brass fitting to the original steel line, being sure the flared end faces forward and tighten. Next, install the braided stainless steel hose to the brass fitting and tighten. (The hose will run under the axle tube and back to the caliper) Then using one of the banjo bolts and two of the crush washers included in this kit, place a crush washer on one of the banjo bolts, the banjo bolt through the brake hose, another crush washer, then into the caliper. And torque the banjo bolts to 32 foot pounds or torque!

The best way to bleed the brakes is to use good ole” gravity! Fill the master cylinder, and open the right rear bleeder screw, and let the brake fluid flow through! This will eliminate finding a second person! After the right side is done, re fill the master cylinder, and repeat the same for the left side. However, once you are done with the gravity bleed method, we do recommend that you get someone to pump the brakes to be sure ALL of the air is gone from the system.

CHECK FOR LEAKS, AND ANY POSSIBLE INTERFERANCE WITH THE BRAKE HOSE’S

Use the enclosed tie wraps to secure the stock steel brake line to the axle tube.

We also recommend that you check your installation each time you drive for the first few days, this is to insure that there are not interferences with the brake hose, lines or emergency brake cables.

YOUR STOCK STUDEBAKER WHEELS WILL NOT CLEAR THE CALIPERS AND BRACKETS! YOU MUST USE EITHER CHRYSLER WHEELS OR FORD WHEELS!!

TURNER BRAKE

672 WALLACE DRIVE

### LITTLE RIVER, SOUTH CAROLINA 29566

www.turnerbrake.com

E-mail diskbrake1@comcast.net

(317) 294-1492

3

#### WARRANTY

## TURNER BRAKE

672 WALLACE DRIVE

LITTLE RIVER, SOUTH CAROLINA 29566

www.turnerbrake.com

(317) 294-1492

This brake kit utilizes, stock American made, modern brake parts from two major automobile manufactures.

This kit is designed to adapt these modern parts to the stock Studebaker spindles, without any permanent modifications to the spindle. The finest quality parts have been assembled for this kit. All of the bolts, lock washers, and lock nuts are of grade 8 aircraft quality, NOTHING LESS WILL DO!

The stock Studebaker caliper-mounting adapter is held on by 3 3/8 x 1 3/4 grade 5 bolts with lock washers. This kit utilizes 4, 3/8 x 1 1/4 grade 8 bolts, with lock washers and lock nuts. BE SURE TO USE RED LOCTITE ALONG WITH THE LOCK WASHERS AND LOCK NUTS!!

Also included in this kit are the proper torque specifications for each set of bolts, THESE MUST ALSO BE USED!!  A competent professional mechanic should install this kit. THE PURCHASER IS RESPONSIBLE FOR PROPER INSTALLATION OF THIS PRODUCT.

LIMITED WARRANTY

TURNER BRAKE, and it's affiliates, warrants to the original purchaser that it's product shall be free from defects in materials and workmanship for a period of ninety (90) days from the date of original purchase. All claims shall be made by returning the product; FREIGHT PRE-PAID, with proof of purchase to TURNER BRAKE.

If found to be defective in material or workmanship upon inspection, TURNER BRAKE will repair or replace the defective product. In no event shall this warranty exceed the original purchase price of this product.

This warranty shall not apply to products subjected to accident, negligence, alteration, abuse, misuse, racing purposes, improper installation, or unsuited uses. Furthermore, TURNER BRAKE shall not be liable for any consequential, special or contingent damages, or injury arising directly or indirectly from any defect in its goods or from use of goods, defective or not.

TURNER BRAKE'S only obligation will be to repair or replace material defects proven to be defective; neither seller nor manufacturer will be liable for any loss, injury, or consequential damages, arising from use or inability to use this product.

IT IS EXPRESSLY UNDERSTOOD AND AGREED THAT THIS WARRANTY SHALL BE IN LIEU OF ALL WARRANTIES OF FITNESS AND IN LIEU OF THE WARRANTY OF MERCHANTABILITY AND THERE ARE NO OTHER WARRANTIES EXPRESSED OR IMPLIED.

4

###### EMERGENCY BRAKE CABLES FOR ALL APPLICATIONS

###### AND ECONOMY KIT!

THERE IS AN EXTRA CHARGE FOR THIS CABLE ON ALL KITS,

We no longer stock the emergency brake cable for use with the TURNER BRAKE rear disc interchange kit. IF you are installing this kit on anything other than an Avanti, you will need to measure the difference in the stock emergency brake cable, and determine what the correct length needs to be. Usually it will need to be shortened, both in total length and the spring at the end of the cable.

You need to take the correct measurements, fill out the following form, and mail it back to TURNER BRAKE. We will forward that measurement that YOU provide to our manufacturer, who will then drop ship the cable back to you.

IF YOU SEND AN INCORRECT MEASUREMENT, WE WILL NOT SEND ANOTHER CABLE! ONLY ONE CABLE WILL BE INCLUDED IN EACH KIT! IF YOU HAVE ANY QUESTIONS ABOUT THIS, PLEASE CONTACT US BEFORE YOU SEND IN THE MEASUREMENTS FOR THE CABLE!

We understand this is a slight inconvenience, but, in trying to accommodate as many different vehicles as possible, we have set this up with our vendor. We expect turn around time of approximately 2 weeks.

TURNER BRAKE

672 WALLACE DRIVE

### LITTLE RIVER, SOUTH CAROLINA 29566

www.turnerbrake.com

(317) 294-1494

TURNER BRAKE

672 WALLACE DRIVE

LITTLE RIVER, SOUTH CAROLINA 29566

www.turnerbrake.com

E-mail diskbrake1@comcast.net

HOW TO MEASURE THE EMERGENCY BRAKE CABLE

We need the measurement between the supports on the frame and the caliper bracket, for the shielded part of the cable.  (A to B)

We also need the total length of the cable from end to end. (C to C) It is easier to use the stock cable and attach it on one caliper, then WITH THE CENTER ADJUSTMENT  (X) AT ITS LONGEST, (nut screwed out all the way to the end of the linkage) Pull the cable until it is SNUG and measure how much will need to be either added or subtracted from your cable. We must know the original length of your cable, and the length you want us to make your new cable.  The measurement from (B to C) is 4 inches. This will not change. Once we receive the back sheet, we will forward it on to the manufacturer. They will drop ship it directly to you.

A                              B                            C

Cable bracket on frame      Cable bracket on caliper   lever on caliper

X center adjustment

A                              B                            C

Cable bracket on frame      Cable bracket on caliper   lever on caliper