PART 1 GENERAL

1.01	GENERAL REQUIREMENTS

- Provide all materials, labor, equipment and services necessary to furnish, deliver and install all work under this section as shown on the contract documents, specified herein, and as specified by the job conditions.

1.02 	DESCRIPTION

- Related work specified elsewhere:

1. Metal Fabrication. 		Section 05 50 00

2. Rough Carpentry. 		Section 06 10 00

3. Access Panels & Doors:	Section 08 31 00

4. Painting:			Section 09 91 00

1.03 	SUBMITTALS

- Procedures: Furnish submittals in accordance with the general requirements specified.

- Shop Drawing: Furnish shop drawings for architect's approval.  Include elevations, sections, and details indicating dimensions, materials, finishes, conditions for anchorage and support of each counter shutter.

- Product Literature: Submit manufacturer's technical literature describing the product to be used under this section.

- Maintenance and Operating Manuals: Furnish complete manuals describing the materials, devices and procedures to be followed in operating and maintaining all counter shutters under this section. Include manufacturer's brochures and parts lists describing the actual materials used in the product.

1.04 	QUALITY ASSURANCE

- Regulatory Requirements: Comply with applicable requirements of the laws, codes, ordinances and regulations of federal, state and municipal authorities having jurisdiction.

- Manufacturer Requirements: Counter shutter manufacturer shall have been in the business of and have experience in manufacturing the type of product covered under this specification section as well as giving credible service for a minimum of five (5) years. Provide list of at least ten (10) completed projects which include the products covered under this section.

1.05 	DELIVERY, STORAGE AND HANDLING

- General: Deliver and store materials in manufacturer's original packaging, labeled to show name, brand and type. Store materials in a protected dry location off the ground in accordance with manufacturer's instructions.

1.06 	WARRANTY

- Counter Shutter Warranty: Provide Two (2) Year Warranty signed by the manufacturer and installer agreeing to repair or replace work which has failed as a result of defects in materials or workmanship. Upon notification within the warranty period, such defects shall be repaired at no cost to the owner.

PART 2 PRODUCTS

  - COILING SECURITY COUNTER SHUTTERS

- Manufacturer: Coiling security counter shutters shall be model CS3000-HK-A as manufactured by McKeon Door Company.

  - MATERIALS

- Curtain: Shall be assembled of interlocking extruded aluminum slats. Slats shall have endlocks locking each end of alternate slats to act as a wearing surface, and maintain slat alignment. Curtain shall be 18 gauge minimum.

  - Slats: Shall be of a cross section not less than 1¼" wide by ½" deep.

- Bottom Bar: Shall consist of a custom aluminum tubular extrusion formed to fit slats. Provide slide locks with hasps on both jambs to allow for locking.

- Guides: Each guide assembly shall be fabricated of a custom aluminum extrusions formed in a box type configuration.

- Mounting Brackets: Fabricated of hot rolled 1/8” steel plate minimum, brackets shall be provided to house ends of the counterbalance barrel assembly.

- Hood: Shall be provided to entirely enclose curtain and counterbalance barrel assembly. Hood shall be fabricated 18 gauge aluminum and designed to match brackets. Top and bottom shall be bent and reinforced for stiffness.

- Counterbalance Assembly: Counter shutter shall be counterbalanced by means of adjustable steel helical torsion springs attached to shaft enclosed in pipe with required mounting blocks or rings for attachment of curtain. Grease sealed bearings or self-lubricating graphite bearings shall be attached to the spring barrel which shall be fabricated of hot formed structural quality carbon steel seamless pipe.

- Hand Crank Operator: Counter shutter shall be provided with a compact reduction geared unit designed as an integral part of the assembly. The hand crank mechanism shall not require more than 35 pounds of operational force to move the counter shutter in either the open or close position.

- Finish: After completion of fabrication, clean all metal surfaces to remove dirt and chemically treat to provide for paint adhesion. All steel components shall receive a coat of prime paint finish and all exposed aluminum shall be of a clear anodized finish.

PART 3 EXECUTION

3.01  	EXAMINATION

- Examine surfaces and field conditions to which this work is to be performed and notify architect if conditions of surfaces exist which are detrimental to proper installation and timely completion of work.

- Verify all dimensions taken at job site affecting the work. Notify the architect in any instance where dimensions vary.

- Coordinate and schedule work under this section with work of other sections so as not to delay job progress.

3.02  	INSTALLATION

- Perform installation using only factory approved and certified representatives of the counter shutter manufacturer.

- Install counter shutter assemblies at locations shown in perfect alignment and elevation, plumb, level, straight and true.

- Adjust counter shutter installation to provide uniform clearances and smooth non-binding operation.

3.03  	PROTECTION AND CLEANING

- Protect installed work using adequate and suitable means during and after installation until accepted by owner.

- Remove, repair or replace materials which have been damaged in any way.

- Clean surfaces of grime and dirt using acceptable and recommended means and methods.