SECTION 08 33 00

COILING SERVICE DOORS (Standard Duty / Low Cycle)

PART 1 GENERAL

1.01
GENERAL REQUIREMENTS

A. Provide all materials, labor, equipment and services necessary to furnish, deliver and install all work under this section as shown on the contract documents, specified herein, and as specified by the job conditions.

1.02 
DESCRIPTION

A. Related work specified elsewhere:

1. Metal Fabrication. 

Section 05 50 00

2. Rough Carpentry. 

Section 06 10 00

3. Access Panels & Doors:
Section 08 31 00
4. Painting:


Section 09 91 00

1.03 
SUBMITTALS

A. Procedures: Furnish submittals in accordance with the general requirements specified.
B. Shop Drawing: Furnish shop drawings for architect's approval.  Include elevations, sections, and details indicating dimensions, materials, finishes, conditions for anchorage and support of each coiling service door.
C. Product Literature: Submit manufacturer's technical literature describing the product to be used under this section.
D. Maintenance and Operating Manuals: Furnish complete manuals describing the materials, devices and procedures to be followed in operating and maintaining all coiling service doors under this section. Include manufacturer's brochures and parts lists describing the actual materials used in the product.

1.04 
QUALITY ASSURANCE

A. Regulatory Requirements: Comply with applicable requirements of the laws, codes, ordinances and regulations of federal, state and municipal authorities having jurisdiction.
B. Manufacturer Requirements: Coiling service door manufacturer shall have been in the business of and have experience in manufacturing the type of product covered under this specification section as well as giving credible service for a minimum of five (5) years. Provide list of at least ten (10) completed projects which include the products covered under this section.

1.05 
DELIVERY, STORAGE AND HANDLING

A. General: Deliver and store materials in manufacturer's original packaging, labeled to show name, brand and type. Store materials in a protected dry location off the ground in accordance with manufacturer's instructions.

1.06 
WARRANTY

A. Coiling Service Door Warranty: Provide Two (2) Year Warranty signed by the manufacturer and installer agreeing to repair or replace work which has failed as a result of defects in materials or workmanship. Upon notification within the warranty period, such defects shall be repaired at no cost to the owner.
PART 2 PRODUCTS

2.01 COILING SERVICE DOORS

A. Manufacturer: Coiling service doors shall be the model SD3022-HC-PC as manufactured by McKeon Door Company.

2.02 MATERIALS

A. Curtain: Shall be assembled of interlocking galvanized steel slats, cold rolled. Slats shall have endlocks locking each end of alternate slats to act as a wearing surface, and maintain slat alignment. Curtain shall be 22 gauge minimum.
1. Slats: Shall be of a cross section not less than 3" wide by 7/8" deep.

B. Bottom Bar: Shall consist of two (2) angles, each not less than 2" x 2" x 1/8" steel formed to fit slats. Provide slide locks with hasps on both jambs to allow for locking.
C. Guides: Each guide assembly shall be fabricated of a minimum 3" x 3" steel support angle, a 2" x 3" inner guide angle and a 3" x 3" outer guide angle.
D. Mounting Brackets: Fabricated of hot rolled 3/16” steel plate minimum, brackets shall be provided to house ends of the counterbalance barrel assembly.

E. Hood: Shall be provided to entirely enclose curtain and counterbalance barrel assembly. Hood shall be fabricated 22 gauge galvanized steel and designed to match brackets. Top and bottom shall be bent and reinforced for stiffness.
F. Counterbalance Assembly: Coiling service door shall be counterbalanced by means of adjustable steel helical torsion springs attached to shaft enclosed in pipe with required mounting blocks or rings for attachment of curtain. Grease sealed bearings or self-lubricating graphite bearings shall be attached to the spring barrel which shall be fabricated of hot formed structural quality carbon steel seamless pipe.
G. Hand Chain Operator: Coiling service door shall be provided with a gear reduction mechanism designed and built by the door manufacturer.  The gear reduction mechanism shall not require more than 35 pounds of operational force to move the coiling service door in either direction. 
H. Finish: After completion of fabrication, clean all metal surfaces to remove dirt and chemically treat to provide for powder coat adhesion. Provide powder coat finish of color as selected by architect from manufacturer’s standard RAL powder coat selection chart.
PART 3 EXECUTION

3.01  
EXAMINATION

A. Examine surfaces and field conditions to which this work is to be performed and notify architect if conditions of surfaces exist which are detrimental to proper installation and timely completion of work.

B. Verify all dimensions taken at job site affecting the work. Notify the architect in any instance where dimensions vary.

C. Coordinate and schedule work under this section with work of other sections so as not to delay job progress.

3.02  
INSTALLATION

A. Perform installation using only factory approved and certified representatives of the coiling service door manufacturer.

B. Install coiling service door assemblies at locations shown in perfect alignment and elevation, plumb, level, straight and true.

C. Adjust coiling service door installation to provide uniform clearances and smooth non-binding operation.

3.03  
PROTECTION AND CLEANING

A. Protect installed work using adequate and suitable means during and after installation until accepted by owner.

B. Remove, repair or replace materials which have been damaged in any way.

C. Clean surfaces of grime and dirt using acceptable and recommended means and methods.

PAGE 3 of 3

