|  |  | SAFETY DATA SHEET |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SECTION 1 –Identification |  |  |  |  |  |
| Product Number and Name | 97007 Wondafix, 30ml pack |  |  |  |  |
| Recommended use | Adhesive |  |  |  |  |
| Manufactured by | Pratley Polymers Manufacturing (Pty) Ltd |  |  |  |  |
|  | Jackson Street, Factoria, Krugersdorp, South Africa, 1745 Tel: +27-11-955-2190 Fax: +27-11-955-3918 www.pratley.com |  |  |  |  |
| Emergency Telephone Number | +27-11-955-2190 (office hours only) |  |  |  |  |
| Sold By | Pratley (Pty) Ltd |  |  |  |  |
| Street address | Jackson Street, Factoria, Krugersdorp, South Africa, 1745 |  |  |  |  |
| Contact details | Tel: +27-11-955-2190 Fax: +27-11-955-3918 www.pratley.com |  |  |  |  |
| Emergency Telephone Number | 011-955-2190 during office hours 021-689-5227 after work hours (Poisons Information Centre) |  |  |  |  |
| SECTION 2 – Hazards Identification |  |  |  |  |  |
| Classification |  |  |  |  |  |
|  | WARNING |  |  |  |  |
| Hazard Statements | UNREACTED MATERIAL |  | Harmful in contact with skin and if swallowed. May cause sensitization by skin contact. Suspected of damaging fertility or the unborn child. Toxic to aquatic life with long lasting effects. |  |  |
|  | REACTED MATERIAL |  | Not classed as hazardous. |  |  |
| Precautionary Statements | Do not handle before all safety instructions have been read and understood. Do not eat drink or smoke when using this product. If swallowed, rinse mouth several times with water. Wear protective clothing and gloves. Wash thoroughly with soap and water after use. Wash contaminated clothing before re-use. Call a poison center/doctor if you feel unwell, are concerned or develop a rash. Avoid release to the environment. Collect spillage. Dispose of in accordance with local regulations. |  |  |  |  |
| Other Hazards | None known. |  |  |  |  |

| SECTION 3 – Composition / Information on Ingredients |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Hazardous Ingredients | % |  | CAS# |  | Name |
| RESIN | 30-50% |  | 25068-38-6 |  | epoxy resin |
|  | 50-65% |  | trade secret |  | blocked isocyanate |
| SECTION 4 – First Aid Measures |  |  |  |  |  |
|  | RESIN |  |  | HARDENER |  |
| Skin | Wash thoroughly with soap and water. If irritation occurs, get medical advice. |  |  |  |  |
| Eye | Rinse cautiously with water for several minutes. Remove contact lenses if present and easy to do so. |  |  |  |  |
| Inhalation | Not likely to be inhaled due to solid nature of the product. If inhaled make sure airways are not blocked and treat as ingestion. |  |  |  |  |
| Ingestion | Do not induce vomiting. Immediately give plenty of water to drink if conscious. Place in the recovery position if vomiting occurs. Seek medical advice. |  |  |  |  |
| SECTION 5 – Fire Fighting Measures |  |  |  |  |  |
| Flammability | Not flammable. |  |  |  |  |
| Auto-ignition Temperature (°C) | Not done |  |  |  |  |
| Suitable extinguishing media | Water fog foam, dry chemical or carbon dioxide. |  |  |  |  |
| Hazardous combustion products | Carbon oxides and traces of other potentially harmful products. |  |  |  |  |
| Protective action for fire-fighters | Wear self-contained breathing apparatus. |  |  |  |  |
| SECTION 6 – Accidental Release Measures |  |  |  |  |  |
| Emergency procedure | Collect and dispose of. |  |  |  |  |
| Protective equipment required | Skin | General purpose gloves and overalls. |  |  |  |
|  | Face / Eyes | Face shield. |  |  |  |
|  | Clothing | No special requirements. Wash clothing thoroughly if contaminated. |  |  |  |
|  | Ventilation | Work in a well-ventilated area. If ventilation is poor use a self-contained breathing apparatus suitable for organic vapours. |  |  |  |
| Environmental precautions | Avoid release to the environment. |  |  |  |  |
| Containment procedure | Scrape up excess adhesive and seal in containers. Soak up any excess with absorbent material. If released into water, immediate collection by a suitably sized scoop is needed. |  |  |  |  |
| Clean-up procedure | Small amounts should be cured by mixing the hardener and resin together and then disposed of in accordance with local regulations. Large amounts would need to be incinerated in accordance with local regulations. |  |  |  |  |
| SECTION 7 – Handling and storage |  |  |  |  |  |
| Handling | Use only as directed. Do not eat, drink or smoke when working with this product. Work in a well-ventilated area. Wash hands with soap and water after use. |  |  |  |  |
| Storage requirements | Store in a cool dry place. Keep out of reach of children. |  |  |  |  |
| Conditions to avoid | Heat and ignition sources. |  |  |  |  |
| Incompatible materials | Oxidizers and strong acids. |  |  |  |  |
| SECTION 8 – Exposure Control / Personal Protection |  |  |  |  |  |
| Exposure Limits | ACGIH TLV |  | Not established |  |  |
|  | OSHA PEL |  | Not established |  |  |
| Engineering controls | None required. Use in a well-ventilated area. |  |  |  |  |
| Personal Protective Equipment | Skin |  | General purpose gloves and overalls should be used. |  |  |
|  | Face / Eye |  | Goggles should be used. |  |  |
|  | Inhalation |  | Inhalation is unlikely due to the nature of the material. If inhaled, treat as ingestion. |  |  |
|  | Ingestion |  | Unlikely to be ingested under normal use. Do not eat while working with this product and keep away from children. |  |  |
|  | Thermal |  | There should be none required under normal use. |  |  |
|  | Other |  | Always wash hands with soap and water after use. |  |  |
| SECTION 9 – Physical and Chemical Properties |  |  |  |  |  |
| Appearance | RESIN: Milky clear viscous liquid HARDENER: Clear yellow viscous liquid MIXED: Off-white flexible solid |  |  |  |  |
| Odour | RESIN: Slight HARDENER: Like ammonia |  |  |  |  |
| Odour threshold | Not done. |  |  |  |  |
| pH | Not done. |  |  |  |  |
| Melting / Freezing point | Not done. This product crystalizes and does not work properly under 10°C. If warmed, it will work properly again. |  |  |  |  |
| Boiling point (initial) | Not done. |  |  |  |  |
| Boiling range | Not done. |  |  |  |  |
| Flash point | Not done. Based on ingredients expected to be >100°C. |  |  |  |  |
| Evaporation rate | Negligible. |  |  |  |  |
| Flammability | Not flammable. |  |  |  |  |
| Flammability limits | Not done. |  |  |  |  |
| Vapour pressure | Not done. |  |  |  |  |
| Vapour density | Not done. |  |  |  |  |
| Relative density | RESIN: 1.8 g/ml HARDENER: 1.0 g/ml |  |  |  |  |
| Solubility | RESIN: Insoluble in water. HARDENER: Partially soluble in water. |  |  |  |  |
| Partition co-efficient: n-octonal / water | Not done. |  |  |  |  |
| Auto-ignition temperature | Not done |  |  |  |  |
| Decomposition temperature | Not done |  |  |  |  |
| Viscosity at 25°C | RESIN: 55000mPa.s HARDENER: 63000mPa.s |  |  |  |  |
| SECTION 10 – Stability and Reactivity |  |  |  |  |  |
| Reactivity | Unlikely to react during normal conditions of use and storage. |  |  |  |  |
| Chemical stability | Stable under normal conditions. |  |  |  |  |
| Possibility of hazardous reactions | Unlikely |  |  |  |  |
| Conditions to avoid | Heat and ignition sources. |  |  |  |  |
| Incompatible materials | Oxidizers and strong acids. |  |  |  |  |
| Hazardous decomposition products | Carbon oxides and traces of other potentially harmful products. |  |  |  |  |
| SECTION 11 – Toxicological Information |  |  |  |  |  |
| Acute toxicity | RESIN: | LD50 (oral – calculated) >7369mg/kg LD50 (dermal – calculated) >1645mg/kg |  |  |  |
|  | HARDENER: | LD50 (oral – calculated) >1507mg/kg LD50 (dermal – calculated) >1548mg/kg |  |  |  |
| Skin corrosion / irritation | Based on human experience, may cause sensitisation. |  |  |  |  |
| Serious eye damage / irritation | No test data. |  |  |  |  |
| Respiratory or skin sensitization | Skin sensitizer based on human experience. |  |  |  |  |
| Germ cell mutagenicity | No information available. |  |  |  |  |
| Carcinogenicity | Does not contain any known carcinogens |  |  |  |  |
| Reproductive toxicity | Contains ingredients that are suspected of damaging fertility or the unborn child. |  |  |  |  |
| STOT – single exposure | No information available. |  |  |  |  |
| STOT – repeated exposure | No information available. |  |  |  |  |
| Aspiration hazard | Unlikely to be aspirated due to the solid nature of the material. |  |  |  |  |
| SECTION 12 – Ecological Information |  |  |  |  |  |
| Acute Toxicity | Category 2 Toxic to aquatic life (based on incorporation of >2.5% Acute Category 2 Toxicity ingredients) |  |  |  |  |
| Chronic Toxicity | No information available. |  |  |  |  |
| Persistence and degradability | Not readily biodegradable. |  |  |  |  |
| Bio-accumulative potential | Has the potential to bio-accumulate from the resin. No information is available for the hardener. |  |  |  |  |
| Mobility in soil | Not mobile in soil. |  |  |  |  |
| Other adverse effects | None known. |  |  |  |  |
| SECTION 13 – Disposal Considerations |  |  |  |  |  |
| Small spills (during personal use) | React the resin and hardener portions together and, once cured, dispose of as normal waste. |  |  |  |  |
| Large spills | Contain and dispose of in accordance with local regulations. |  |  |  |  |
| Special precautions- incineration | May release carbon oxides and traces of other harmful products. |  |  |  |  |
| Special precautions for landfill | Avoid release to the environment. |  |  |  |  |
| Disposal container | None specified. |  |  |  |  |
| SECTION 14 – Transport Information |  |  |  |  |  |
| UN Number | UN3082 |  |  |  |  |
| UN Proper Shipping Name | ENVIRONMENTALLY HAZARDOUS LIQUID, N.O.S. Epoxy resin |  |  |  |  |
| Transport Hazard Class | 9 |  |  |  |  |
| Packing Group | III |  |  |  |  |
| Environmental Hazard | Category 2 Acute. |  |  |  |  |
| Special precautions | None known. |  |  |  |  |
| MARPROL 73/78 | Not applicable as not transported in bulk. |  |  |  |  |
| IBC Code | Not applicable as not transported in bulk. |  |  |  |  |
| SECTION 15 – Regulatory Information |  |  |  |  |  |
| Montreal Protocol | Not available. |  |  |  |  |
| Stockholm Convention | Not available. |  |  |  |  |
| Rotterdam Convention | Not available. |  |  |  |  |
| Safety Regulations | If medical advice is needed, have product container or label at hand. Keep out of reach of children. Read label before use. |  |  |  |  |
| Health Regulations | UNREACTED MATERIAL Harmful in contact with skin and if swallowed. May cause sensitization by skin contact. Suspected of damaging fertility or the unborn child. REACTED MATERIAL is not classed as hazardous. |  |  |  |  |
| Environmental Regulations | Resin is toxic to aquatic life with long lasting effects. |  |  |  |  |

| SECTION 16 – Other Information |  |  |
| --- | --- | --- |
| Date prepared | 2014.10.13 |  |
| Revision number | 03 |  |
| Changes from previous version | Updated to GHS system. Separated from other epoxy products. Re-evaluation of hazards. |  |
| Key to abbreviations | UN | United Nations |
|  | IBC | International Code for the Construction and Equipment of Ships carrying Dangerous Chemicals in Bulk (International Bulk Chemical Code) |
|  | Montreal Protocol | Montreal Protocol on Substances that deplete the Ozone Layer |
|  | Stockholm Convention | Stockholm Convention on Persistent Organic Pollutants |
|  | Rotterdam Convention | Rotterdam Convention on the Prior Informed Consent Procedure for Certain Hazardous Chemicals and Pesticides in International Trade |